Mach64 Accelerator Programmer's Guide Sample Code Disk

(c) Copyright 1993-94 ATI Technologies Inc.  All rights reserved.

March 7, 1994

UNPACKING SAMPLE CODE:

Run the INSTALL batch file on the Sample Code disk and follow its

instructions. Each code example has its own subdirectory.

COMPILING:

To compile any of the code examples, change to the example's directory and

type MAKE or a similiar makefile utility. To compile all of the examples,

change to the SAMPLE directory and type COMPILE. The COMPILE batch file

assumes the makefile utility name is MAKE. This will force an entire

compile using the current C compiler and macro assembler. The .obj and .exe

files included were compiled by the Microsoft C compiler (v6.00) and

assembler (v5.10). The C source code is fairly generic and should allow

compiling by other compilers with relatively little modification. Different

compiler and linker switches may be required for non-Microsoft products.

See the makefile for any sample code example for switch information.

EXECUTING:

To execute any of the code examples, change to the example's directory and

type the executable filename. To run all of the examples in sequence

(alphabetically), change to the SAMPLE directory and type RUN. Note that

most of the sample code examples require keyboard input (i.e. press a key)

to step through the programs and to exit.

SAMPLE CODE DESCRIPTION:

ABLIT

This is sample code to show usage of the VGA 32K apertures for transferring

pixel data from one region to another. This particular example uses the

apertures to transfer one line at a time and shows the paging calculations.

There are two write and two read 32K apertures. They may be used separately

or combined to effectively make one 64K write aperture and one 64K read

aperture. The combined approach has the advantage of handling 'overflows' if

the lower 32K aperture size is exceeded.

BMODE

This is sample code to set an accelerator mode using the Mach64 ROM services.

The screen contents are not cleared and any screen information left from

a previously operation will be shown.

CHAIN

This is sample code to demonstrate chained context operations. Two context

operations (a filled rectangle and a line) are chained together to form one

"atomic" operation.

COMPARE

This is sample code to show the effect of mixing and the comparing of the

source with the destination.

CURSOR

This is sample code to setup a hardware cursor bitmap. The hardware cursor

is enabled and shown in the center of the screen. This is a static example.

DBUF

This is sample code to show image drawing using double buffering. This

example uses 8 targa files located in the IMAGE directory for its source

images. The system timer is used to switch buffers and therefore gives a

fairly slow frame rate. A faster switch source (such as one used in the SBUFI

example) may be used to obtain faster frame rates.

FIXPAT

This is sample code to show the three fixed patterns available on the

Mach 64. Note that the 4x2 and 8x1 pattern modes are only available in

8 bpp accelerator modes. The other (8x8 monochrome pattern) performs mono

expanded patterned fills.

GPBLIT

This is sample code to show a general pattern blit. The source is replicated

several times in both the x & y axis to produce the destination. This is

accomplished by source wrapping in both axis.

GPLINE

This is sample code to show drawing a line using a pattern (general pattern

line). The source line pattern is read in both directions (left-right and

right-left) to produce two destination lines.

GPRBLIT

This is sample code to show a general pattern blit with rotation. It is

similiar to GPBLIT except the first destination rectangle drawn is from a

partial region of the source.

LINAPP

This is sample code to show usage of the big linear aperture. A buffer of

data in CPU memory will be copied to video memory via the linear aperture

and then read back. The contents are dumped to the screen.

LINE

This is sample code to draw a line. The engine does not support lines in

24 bpp modes. Each line octant is shown in a different color.

MEBLIT

This is sample code to show a monochrome to color expansion blit. The

character 'a' is drawn from a monochrome bitmap.

PALETTE

This is sample code to initialize the palette in 8 bits per pixel modes. The

entire palette is shown in the form of 256 filled rectangles, each

representing one of the palette entries.

PAN

This is sample code to show panning of a 1024x1024 image in a 640x480

viewable screen area. The following keys are available for panning: the

four arrow keys, HOME, and END. The ESC key will exit the program.

POLYGON

This is sample code to show how to setup and fill a polygon shape. Polygons

are not supported in 24 bpp modes. The shape filled is a pentagon.

RECT

This is sample code to draw a filled rectangle. Several different sized

filled rectangles are drawn.

REGMAP

This is sample code to show how to access the memory mapped registers using

the VGA and linear apertures.

SBLIT

This is sample code to show a simple one-to-one blit.

SBUFI

This is sample code to show image drawing using vertical line interrupts.

This example uses 8 targa files located in the IMAGE directory for its source

images. Note that this example will not work if the interrupt jumper is not

in place (this may not applicable for all cards). IRQ2 is assumed for all

bus types except PCI. For PCI, select IRQ10 to be available in the system

BIOS setup.

SBUFP

This is sample code is similiar to SBUFI except it is uses vertical line

polling instead of interrupts.

SCISSOR

This is sample code to setup the engine scissors. An "scissored" rectangle

is drawn over an "unscissored" rectangle.

STYLE

There are three examples under this heading. Each one illustrates one of the

three programming styles described in the programmer's guide. A filled

rectangle is drawn for each one (a different color).

TBLIT

This is sample code to show a transparent blit. Transparent blits are drawn

over a multi-colored filled rectangle background.

WMASK

This is sample code to show the effects of the DP_WRITE_MASK register.